Steps for Using JADE Program for MSE360

Lab 3, Week 1

Open Program

- On desktop, double-click on JADE 3.1 icon

- Click button for Load <some filename>

Open your File

- At top, click on File, then Open (or CTRL-O)

- Find your file and select it

- Click on New Plot

Smoothing Data

- At top, click on Filters, then Smooth Scan Trace (or F5)

- In dialog box in top left of screen, choose number of points (for example, 15), and make sure that the box for Less Smooth on Peak Tops is checked

- At top, click on Analyze, then Take New Pattern

Removing Background Signal

- At top, click on Analyze, then Fit Background (or F3)

- In dialog box, choose Polynomial 1, then click OK

- At top, click on Analyze, then Remove Background (or F4)

Getting List of Peaks

- At top, click on Analyze, then Find Peaks (or F2)

- In dialog box, click OK

- (Optional step) To add peaks manually, zoom in on an area in the spectrum near the top of the screen by drawing a box around it.  Then click on the 3rd button from the left (Peak Edit II Cursor), and move the mouse on top of the peak (in the zoomed in view), and click the mouse.  To go back to the normal zoom, click on the button in the far bottom right corner (Full Range or Refresh View)

- Once all peaks are identified, at the top click on View, then Report & Printer, then Peak List Report

- Print this list or save it as a PID file

Saving Spectrum as a TIF File

- At top, click on File, then Print Setup

- At top left, click on Menu, then Copy to Image File

- In dialog box, give it a filename and click OK